# Graduation Thesis — Draft (FAE Template)

Working draft for the HCMUTE Faculty of Advanced Education graduation thesis, structured to match doc/ref/FAE Template of Graduation Thesis.pdf. Content is derived from the actual repository (modules/, config.json, doc/ai_context.md, doc/theory_basis.md, doc/academic_report.md). Markers [TODO: …] flag values that must come from real measurements or from the student before submission — do not leave these in the final document, and do not invent numbers to replace them.

## Cover Page (fill in before export)

- HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY AND ENGINEERING — FACULTY OF ADVANCED EDUCATION

- GRADUATION PROJECT

- Title: VISION-GUIDED DELTA ROBOT FOR REAL-TIME PICK-AND-PLACE SORTING ON A CONVEYOR

- Supervisor: [TODO: supervisor name, title]

- Full name(s): [TODO]

- Student ID(s): [TODO]

- Major: AUTOMATION AND CONTROL ENGINEERING TECHNOLOGY

- Ho Chi Minh City, [TODO: month] 2026

## DISCLAIMER

This graduation report is submitted in partial fulfillment of the requirements for the bachelor’s degree program at Ho Chi Minh City University of Technology and Education. The work presented in this report is the result of my own independent research, study, and practical implementation, conducted under the academic supervision of the assigned lecturer.

All data, figures, methodologies, and results presented in this report have been obtained through legitimate academic activities. Any sources of information, ideas, or materials taken from other works have been appropriately cited and referenced in accordance with academic standards. The author confirms that this report has not been submitted, in whole or in part, to any other institution for the purpose of obtaining any academic degree or qualification.

The opinions, analyses, findings, conclusions, and recommendations expressed in this report are solely those of the author and do not necessarily reflect the official views of Ho Chi Minh City University of Technology and Education, the Faculty of Advanced Education, or the project supervisor.

## ACKNOWLEDGEMENTS

[TODO: personalize.] I would like to express my sincere gratitude to my supervisor, [TODO: supervisor], for the valuable guidance, continuous support, and constructive feedback throughout this graduation project. I also thank the lecturers of the Faculty of Advanced Education for the solid foundation in control systems, robotics, and computer vision that made this work possible, and my family and friends for their constant encouragement.

## ABSTRACT

Manual sorting of small electronic components on a production line is repetitive, error-prone, and difficult to scale. This thesis presents the design and implementation of a vision-guided delta parallel robot that performs real-time pick-and-place sorting of surface-mount package components (QFP and TQFP) directly from a moving conveyor belt. A USB camera streams 1080p frames that are processed on a host PC by a YOLO26 oriented-bounding-box (YOLO26-OBB) detector, which returns each component’s class, centroid, and tilt angle; a locator-dot marker resolves the full 360° heading needed to align the suction cup. Detected components are mapped from image space to the robot workspace through a homography and a planar homogeneous transform, and are tracked drift-free by anchoring each object to the conveyor’s absolute encoder position.

The control software runs as a concurrent Python application that isolates network latency in a dedicated communication process and senses the belt and vision pipeline in a high-frequency perception thread, while a main decision thread plans jerk-bounded trajectories and executes picks. Motion is delegated to an Omron NX1P2 PLC over EtherNet/IP, while conveyor speed and the 4th-axis suction rotation are commanded on a Siemens S7-1200 PLC over the S7 protocol. A positional pick gate triggers the descent on the live tracked position of the target rather than a predicted time, making interception immune to belt-speed estimation noise. An optional adaptive conveyor-speed controller regulates the belt inversely to product density to keep the arm fed at a stable presentation rate.

Software-in-the-loop validation of the scheduler — exercising the real planning, tracking, and timing code with simulated motion execution in place of the PLC — reached the three fixed accuracy targets to within 0.01 mm and completed 12/12 simulated picks across a two-cycle evaluation run at a software-timing ceiling of approximately 38 picks/min. Experimental evaluation on the physical cell measured [TODO: pick success rate]% pick success, a positioning accuracy within [TODO: mm], and a sustained throughput of [TODO: picks/min]. The results demonstrate a cost-effective architecture that bridges real-time computer vision and high-speed parallel-robot motion control for flexible small-component sorting.

Keywords: Delta robot, Parallel kinematics, Pick-and-place, Conveyor tracking, YOLO26-OBB, Oriented object detection, Computer vision, PLC, EtherNet/IP, Real-time control, Python.

# Chapter 1: INTRODUCTION

## 1.1. Research Background

Flexible automation of pick-and-place tasks is a core problem in modern electronics manufacturing. Delta (parallel) robots are widely used for high-speed handling because their actuators remain mounted on the fixed base rather than traveling with the end-effector, giving the moving structure low inertia and therefore the high acceleration and repeatable precision that serial-arm manipulators cannot match at comparable speed [2][3][4]. When such a robot must pick objects from a moving conveyor instead of a fixed feeder tray, two problems must be solved together: (i) perceiving where each object is, what type it is, and how it is oriented; and (ii) intercepting it reliably despite communication latency and variation in belt speed — neither problem is solved by the kinematic model alone, which assumes the target position is already known and static.

Recent advances in real-time deep-learning object detection — particularly the YOLO family of single-stage detectors — make on-line perception practical on commodity hardware without a dedicated machine-vision controller [5][6]. Standard axis-aligned bounding boxes, however, only localize an object’s position and extent; they say nothing about its rotation. Oriented bounding box (OBB) detection additionally recovers object heading [5][7], which is essential when a suction tool must be aligned to a rectangular package such as a QFP/TQFP integrated circuit before it can be placed correctly downstream. This project combines a YOLO26-OBB [1] perception pipeline with a delta robot and an industrial PLC motion layer (§2.3) to build a complete vision-guided sorting cell, addressing the perception, tracking, and timing problems explicitly rather than assuming a stationary, pre-localized target.

## 1.2. Research Objectives

### 1.2.1. Real-time vision-guided interception

Detect QFP/TQFP components on a moving belt, resolve their full 360° orientation, track them without drift, and pick them while the belt is moving.

### 1.2.2. Robust, latency-tolerant pick timing

Replace fragile time-of-arrival prediction with a closed-loop positional pick gate so that interception remains accurate under belt-speed fluctuation and network jitter.

### 1.2.3. Throughput regulation

Provide an optional adaptive conveyor-speed policy that keeps the rate-limited robot arm fed at a stable presentation rate under an irregular upstream feeder.

## 1.3. Scope of Research

### 1.3.1. Object and processing environment

- Targets: two surface-mount package classes, QFP (30 × 40 mm) and TQFP (40 × 50 mm), 1.6 mm thick, presented flat on a conveyor belt.

- Robot: a 3-arm delta mechanism with a 4th degree of freedom (suction-cup rotation).

- Perception: a single fixed USB camera (1080p, MJPEG, ~30 fps) viewing the belt upstream of the robot workspace.

### 1.3.2. Focus on engineering and technology

The thesis emphasizes systems integration: the kinematic model, the perception-to-robot coordinate chain, the concurrency architecture, the PLC communication contracts, and the real-time scheduling logic. It is an applied engineering project rather than a study of a new learning algorithm.

### 1.3.3. Limitations of the project

- Suction success is not sensed; a missed grip is not retried (each object is attempted once).

- The 4th-axis firmware ignores per-segment timing; PC-side segment times are approximations.

- Adaptive belt-speed is opt-in and was under hardware testing at the time of writing.

- The camera uses a fixed manual exposure (exposure_time_absolute=300, auto-exposure disabled) to keep capture frame rate constant; this trades off automatic adaptation to ambient lighting changes, so detection quality is only guaranteed under the lighting condition the exposure value was tuned for, and re-tuning is a manual step if the cell is relocated.

- The vision→conveyor homography and conveyor→robot transform (§2.5) are calibrated once for a fixed camera mounting and conveyor alignment; physically moving the camera, the belt, or the robot base requires re-calibration — there is no online recalibration mechanism.

- [TODO: add any additional calibration / lighting / camera limitations observed in practice on the physical cell.]

### 1.3.4. Fundamental values and development direction

The cell demonstrates a reusable pattern — vision perception → coordinate transform → encoder-anchored tracking → positional-gate interception — that generalizes to other flexible sorting and assembly tasks.

## 1.4. Report Structure

- Chapter 1 — Introduction: background, objectives, scope.

- Chapter 2 — Theoretical Basis: delta kinematics, coordinate transforms, oriented object detection, trajectory generation, conveyor tracking, and the adaptive-speed model.

- Chapter 3 — System Configuration: methodology, hardware, and software implementation.

- Chapter 4 — Experiment and Evaluation: test setup and measured results.

- Chapter 5 — Conclusion and Future Work.

# Chapter 2: THEORETICAL BASIS

## 2.1. Overview of the Delta Parallel Robot

The delta robot is a parallel manipulator in which three identical arms connect a fixed top base to a moving end-effector platform through parallelogram forearms, constraining the platform to pure translation in the negative-Z workspace. Because the heavy actuators remain on the base, the moving mass is small, enabling the high accelerations that make delta robots the standard choice for high-speed pick-and-place. The kinematic foundations used here follow standard robotics references [2][3][4]. Geometric parameters of the implemented robot:

| Symbol | Meaning | Value |
| --- | --- | --- |
| sb | base equilateral-triangle side | 320.0 mm |
| sp | end-effector platform side | 94.0 mm |
| L | bicep (upper arm) length | 140.0 mm |
| l | forearm link length | 315.0 mm |

All kinematics, trajectory interpolation, and servo command generation execute on the Omron NX1P2 PLC in Structured Text, inside a 4 ms hard real-time periodic task synchronized to the EtherCAT bus (Distributed Clocks enabled, three MADLN05BE servo drives in CSP — Cyclic Synchronous Position — mode, PDO cycle 4 ms). The host PC never computes a joint angle: it sends Cartesian waypoints and reads back Cartesian/joint telemetry that the PLC itself derives. This split exists because Distributed-Clock-synchronized, sub-microsecond-jitter axis commands are required to keep the three mechanically coupled arms from binding against each other — a timing guarantee a general-purpose OS cannot provide over a host-PC network link.

## 2.2. Kinematics

### 2.2.1. Inverse kinematics (IK)

Given a Cartesian target X0,Y0,Z0, the joint angles θ1,θ2,θ3 are found by exploiting the 120° symmetry: the 3D problem reduces to three identical 2D single-arm solvers in the local YZ-plane (Calc_Angles_YZ), each called once per arm by a top-level coordinator (Calc_Inverse_Kinematics).

Single-arm 2D solver. The base joint of the reference arm sits at y1=−sb/23 and the platform hinge, offset by the target Y0, at ytmp=Y0−sp/23. The elbow 0,yj,zj must simultaneously lie on the bicep circle of radius L centered at 0,y1,0 and on the forearm sphere of radius l centered at X0,ytmp,Z0:

yj−y12+zj2=L2  X02+ytmp−yj2+Z0−zj2=l2

Subtracting the expanded bicep equation from the expanded forearm equation cancels both quadratic terms (yj2, zj2) and linearizes zj in terms of yj:

zj=a+b yj,  a=X02+ytmp2+Z02+L2−l2−y122Z0,  b=y1−ytmpZ0

Substituting back into the bicep circle gives a quadratic Ayj2+Byj+C=0 with A=1+b2, B=2ab−y1, C=y12+a2−L2. Using the half-discriminant d=Δ/4=−a+by12+L2b2+1 avoids one factor of 2 in the root formula; d<0 means the target is outside the reachable dome (the PLC aborts the move). The two roots are the “elbow-up” and “elbow-down” assembly configurations; the physically installed geometry always uses the elbow-down (far, lower) branch:

yj=y1−ab−db2+1,  zj=a+b yj

The joint angle is the bicep’s angle from horizontal, θ=arctan​−zjy1−yj, with a 180° quadrant correction when yj>y1 (atan only returns −90°,90°) and a direct ±90° assignment when yj=y1 to avoid a division by zero.

Per-arm rotation. Arms 2 and 3 are mounted at +120° and −120° from arm 1. Rather than re-deriving the solver three times, the target X,Y is rotated into each arm’s local frame and fed through the same 2D solver:

θ1=SolverX,Y,Z

X2=Xcos120°+Ysin120°, Y2=−Xsin120°+Ycos120°, θ2=SolverX2,Y2,Z

X3=Xcos120°−Ysin120°, Y3=Xsin120°+Ycos120°, θ3=−SolverX3,Y3,Z

θ3 is negated because the third servo drive’s parameter direction is inverted relative to axes 1–2, so a positive encoder count on axis 3 would otherwise correspond to downward rather than upward arm motion. A joint-limit check (θi<−20°) additionally rejects targets that would swing an arm above horizontal into the base frame. All three angle solves plus the limit checks complete in under 10 µs on the NX1P2 — negligible against the 4 ms EtherCAT budget.

### 2.2.2. Forward kinematics (FK)

FK runs every scan (Calc_Forward_Kinematic, fed by the live servo encoders) purely as a position monitor — it is not in the motion-command path. Given joint angles θ1,θ2,θ3, each elbow is projected into 3D using the radial offset w=sb−sp/23:

J1=0−w+Lcosθ1−Lsinθ1 J2=w+Lcosθ2cos30°w+Lcosθ2sin30°−Lsinθ2 J3=−w+Lcosθ3cos30°w+Lcosθ3sin30°−Lsinθ3

The platform center P=X0,Y0,Z0 lies a forearm length l from every elbow: ∥P−Ji∥2=l2. Subtracting sphere 1 from spheres 2 and 3 cancels the shared quadratic terms X02+Y02+Z02, leaving a 2×2 linear system in X0,Y0 parameterized by Z0, solved by Cramer’s rule for X0=a1+b1Z0, Y0=a2+b2Z0. Back-substituting into the sphere-1 equation gives a quadratic in Z0 alone:

AqZ02+BqZ0+Cq=0,  Aq=b12+b22+1

whose lower root, Z0=−Bq−Bq2−4AqCq2Aq, is the physical solution because the workspace is entirely below the base plane; the upper root is geometrically valid but mechanically unreachable. A negative discriminant means the three encoder readings describe a geometrically impossible configuration (mechanical fault or encoder slip) and is treated as a safety fault.

### 2.2.3. Worked numeric example (IK → FK round trip)

To make the model concrete, one of the deployed accuracy test points from config.json (scheduler.accuracy_points[0] = 66.64, −52.9, −290.0 mm) is solved by hand with the exact formulas above (sb=320, sp=94, L=140, l=315 mm). For arm 1: y1=−92.376 mm, ytmp=−80.036 mm, a=−11.704, b=0.0426, d=19391.0 (reachable, d>0), giving elbow yj,zj=−230.712, −21.522 mm and θ1=8.843°. Repeating for the ±120°-rotated targets gives θ2=15.046° and θ3=−44.418°. Feeding these three angles back through the FK equations of §2.2.2 returns X0,Y0,Z0=66.640, −52.900, −290.000 mm — recovering the original target to better than 10−3 mm, confirming the IK and FK derivations are mutually consistent.

## 2.3. Motion Layer: PLC Control, EtherCAT, and Trajectory Execution

Motion is executed by industrial controllers rather than a microcontroller, split by real-time requirement: - An Omron NX1P2 PLC drives the three delta-arm servo axes and the vacuum pump/valve over EtherCAT (three MADLN05BE drives, CSP mode, 4 ms PDO cycle, Distributed Clocks). - A Siemens S7-1200 PLC drives the conveyor speed and the 4th-axis suction-cup rotation over the S7 protocol (TCP).

This split keeps deterministic, safety-critical motion on hardened controllers while the host PC performs perception and high-level scheduling. The byte-level data contracts (DB offsets, endianness, command IDs) are documented in doc/ai_context.md §3.

### 2.3.1. PC ↔ Omron command protocol

The PC writes a commandID (2 = goto-absolute, 3 = go-trajectory, 4 = home, 5 = pick, 6 = release) plus argument arrays to a shared DB; the PLC’s command dispatcher (Section0, rung 4) executes the command within one scan and acknowledges by clearing commandID/bit_doing, reporting task_doing/task_state (1 = Done, 2 = Busy, 3 = Error) back to the PC. Command 3 loads a 7-point waypoint array (X,Y,Z plus an end-effector action bit E∈{0,1} for pump on/off per point) that drives the trajectory pipeline of §2.7.

### 2.3.2. Homing and safety interlocks

On power-up, MC_Home_Delta drives all three arms upward until limit switches trigger, then a calibration move sets each axis to a fixed reference angle (28.9°, 27.5°, −27.7°) before the encoder origin is latched — establishing the absolute coordinate system used by every IK/FK call thereafter. A homing-limit-switch trip outside of an active homing sequence triggers an immediate MC_Stop on all three axes and clears any in-flight trajectory state, and the physical Reset button clears servo faults but forces the operator to re-enable and re-home before further motion.

## 2.4. Computer Vision and Oriented Object Detection

### 2.4.1. Object detection background

Object detection has evolved rapidly from hand-crafted features to deep one-stage detectors; a comprehensive history is given by Zou et al. [5]. The YOLO single-stage family is the de-facto standard for real-time industrial detection [6]. This project uses YOLO26 [1], the 2026 Ultralytics release whose end-to-end (NMS-free) design and removal of the distribution-focal-loss module target low-latency edge inference.

### 2.4.2. Oriented bounding boxes (OBB)

A rectangular IC package has a meaningful in-plane rotation, so a standard axis-aligned box is insufficient. Oriented detection regresses a rotated box; the design rationale and accuracy of oriented detectors are established by works such as Oriented R-CNN [7]. The deployed model is a YOLO26-OBB (nano) weight trained on QFP/TQFP packages, run at 1920 px input with a confidence threshold of 0.7.

### 2.4.3. 360° yaw resolution

YOLO-OBB reports tilt only within [−90°,90°), which is ambiguous for a 180°-symmetric package. A small locator-dot marker on pin/corner #1 disambiguates heading: the vector from the component centroid to the marker centroid gives θheading=atan2vy,vx⋅180/π+θoffset mod 360.

## 2.5. Coordinate Frames and Spatial Transformations

Three frames link the camera to the robot:

- Vision frame (V): pixel coordinates px,py.

- Conveyor frame (C): belt-aligned u,v, with +u downstream.

- Robot frame (R): Cartesian XR,YR,ZR, workspace in Z<0.

A planar homography H maps V→C, and a 2D homogeneous transform F maps C→R at the constant pickup height:

XRYR1=Fuv1=−sinθcosθTXcosθsinθTY001uv1

where θ is the conveyor-to-robot angle (28° in the deployed cell) and TX,TY the conveyor origin offset.

## 2.6. Conveyor Tracking and the Positional Pick Gate

### 2.6.1. Encoder-anchored dead reckoning

Integrating velocity over time accumulates drift and fails when the belt accelerates or stops. Instead, each object is anchored to the absolute belt encoder pt at detection time: ut=uanchor+pt−panchor. Position then tracks physical belt displacement exactly, with zero drift.

### 2.6.2. Positional pick gate

The arm first parks at a kinematically feasible interception point upick, found by a fixed-point iteration over the predicted pick time (guess a lead time → project the object at vbelt → compute the robot’s goto travel time → refine the guess → repeat to convergence), with a minimum 1.6 s lead (intercept_lead_time_s) and edge-clamping for objects already near the danger zone. Once parked, the descent does not fire on that predicted time; the main thread instead waits on the claimed object’s live, continuously refreshed position and fires the moment

ut≥upick−offsetvbelt

Closing the loop on the live object — rather than trusting a frozen, noisy speed sample — removes the mistimed “arrive → wait → miss” lag that a pure time-based trigger exhibits under belt-speed estimation error.

The lead offset, in general. Because u is anchored to the belt encoder, the offset is exactly the belt’s own displacement over the dispatch-to-grip latency Tdelay≈robot_movement_delay_s+ethernet_delay_s≈50 ms. Three cases arise depending on whether the belt is mid-acceleration-ramp when the gate is evaluated (current speed vc, setpoint vsp, ramp duration Taccel=vsp−vc/anom):

offset=vbelt Tdelaysteady belt a=0vc Tdelay+12a Tdelay2mid-ramp, Taccel≥Tdelayvc+vsp2 Taccel+vsp Tdelay−Taccelmid-ramp, Taccel<Tdelay

All three forms are continuous (they agree at Taccel=Tdelay and reduce to the steady-belt case at a=0). In the deployed system the speed-change commit instant is engineered (§2.8) so that any acceleration ramp has already settled by the time the next gate is evaluated; the gate therefore always uses the simple steady-belt form vbelt Tdelay (≈2.5 mm at 50 mm/s, growing with speed), and the two acceleration-aware forms exist only as a robustness fallback should a gate ever fire while a ramp is still in progress (doc/theory_basis.md §3.1, §6.4).

## 2.7. Trajectory Generation

Every pick uses a 7-point goto template and a 7-point pick template — lift → cruise at clearance height → slope down → intercept → lift → transfer → place — with a strict height hierarchy clearance > slope_transition > pre_pick > pickup enforced for safety. The PC only computes the seven Cartesian waypoints (plus an end-effector pump bit per point) and dispatches them as the 7-point array of §2.3.1; all interpolation between waypoints, profile selection, and IK conversion happens on the PLC, inside six daisy-chained instances of a custom MC_Inter_Curve_Vel function block — one per segment of the 7-point path — executing every 4 ms.

### 2.7.1. Profile selection: S-curve vs. trapezoidal

Each segment picks one of two velocity profiles depending on its boundary velocities:

| Vstart | Blend mode | Profile | Why |
| --- | --- | --- | --- |
| =0 | off (final segment) | S-curve polynomial | both boundaries at rest — closed-form, no iteration |
| >0 | on (segments 0–4) | Trapezoidal + look-ahead | non-zero boundary velocity makes the S-curve’s 4-coefficient boundary-value problem require an unbounded-iteration numerical solve for the unknown segment duration T — infeasible inside a 4 ms watchdog-timed scan |

The S-curve is therefore reserved for the one rest-to-rest segment (the final point of each template), while the five continuously-blended segments use the trapezoidal profile, whose duration tacc=Vpeak−Vstart/Amax is a single closed-form division — deterministic, sub-microsecond, and immune to the watchdog-trip risk an iterative S-curve solve would carry.

### 2.7.2. Jerk-bounded S-curve (rest-to-rest segment)

For Vstart=Vend=0, the position profile is the 4th-order Hermite smoothstep Sτ=Vmaxtaccτ3−12τ4 with τ=t/tacc, giving velocity vτ=Vmax3τ2−2τ3 and acceleration aτ=6Vmaxtaccτ1−τ — zero at both ends, peaking at τ=0.5. Why the 1.5× shape factor: a constant-acceleration ramp reaches Vmax in tlin=Vmax/Amax, but this bell-shaped acceleration curve peaks at only 1.5Vmax/tacc for the same tacc — i.e. to respect the same Amax ceiling it needs 50 % longer:

1.5Vmaxtacc=!Amax⟹tacc=1.5 VmaxAmax

This finite, bounded jerk (vs. the trapezoidal profile’s infinite jerk at phase corners) is reserved for the segment where the end-effector is stationary at a pick/place point — exactly where vibration would otherwise corrupt positioning accuracy.

### 2.7.3. Blended corner velocity (look-ahead)

At a non-stop waypoint B between segments AB and BC, the exit velocity into the next segment is capped by the turn angle Θ between v1=B−A and v2=C−B:

cosΘ=v1⋅v2∥v1∥∥v2∥,  Vcorner=Vmaxcos​Θ2=VmaxcosΘ+12

using the half-angle identity — straight-through (Θ=0°) keeps full speed, a 180° reversal forces a full stop. The actual commanded exit velocity additionally clamps against the distance-limited reachable velocity from Vstart over the segment length L, Vreach=Vstart2+2AmaxL, and the global cap Vmax: Vend=minVcorner,Vreach,Vmax — this becomes the next segment’s Vstart, chaining a continuous velocity profile across all six segments.

## 2.8. Adaptive Conveyor-Speed (Rate Regulation)

The belt is treated as a rate regulator, not a speed maximizer: the serial, rate-limited robot arm is the valuable resource, and the upstream feeder is bursty, so the belt’s job is to keep the arm fed at a steady presentation rate rather than to run as fast as possible. An earlier design that raised belt speed with density (v=A⋅N+vmin) had the sign backwards — speeding up under high density shortens each object’s transit time and pushes more objects past the workspace unpicked. The corrected model regulates a rate with a single law.

### 2.8.1. Belt kinematics and bounds

| Symbol | Meaning | Deployed value |
| --- | --- | --- |
| vmin | operational floor (belt_speed_min_mm_s) — hardware control is imprecise below ~20 mm/s | 30 mm/s |
| vcap | pickability ceiling, derived (§2.8.3) | ≈ 109.4 mm/s |
| vhw,max | hardware effective maximum (belt_speed_hw_max_mm_s) | 200 mm/s |
| anom | nominal belt acceleration (belt_accel_mm_s2) | 22.31 mm/s² |
| Tdelay | dispatch→grip latency (§2.6.2) | ≈ 50 ms |
| μmax | robot maximum pick rate =1/tpick (pick_cycle_s, calibrated) | 0.50 obj/s |
| k | headroom factor (belt_speed_headroom, deployed config — the code default is 0.75, but the live config.json value is lower) | 0.6 |
| λnom | nominal presentation-rate target =k μmax | 0.30 obj/s |

### 2.8.2. Inverse-density control law

To hold the arm’s presentation rate λ=ρv (linear density × belt speed) at λnom, the belt speed is set inversely to the measured linear density ρ=N/Lmeas of unclaimed, not-yet-passed objects. L_meas defaults to umax=363 mm — the distance from the conveyor-frame origin (ROI origin) to the far edge of the pick workspace — unless overridden by belt_density_length_mm (scheduler.py:969):

v=clamp​λnomρ, vmin, vcap=clamp​λnom LmeasN, vmin, vcap

— hyperbolic, not linear, in N: more density ⇒ slower belt (more transit time for the serial arm); a sparse feeder ⇒ faster belt (so the arm does not starve). When N=0 the implementation short-circuits to vcap directly rather than dividing by zero (scheduler.py:970-971).

### 2.8.3. Pickability ceiling and worked density example

vcap uses the pick workspace window length L=umax−umin=363−188=175 mm (distinct from Lmeas above) and the minimum transit time ttransit=1.6 s (pick_transit_min_s):

vcap=minL/ttransit, vhw,max=min175/1.6, 200≈109.4 mm/s

With λnomLmeas=0.30×363≈108.9, the law v≈108.9/N gives, for increasing density:

| N (objects in window) | v before clamp | Clamped v | Regime |
| --- | --- | --- | --- |
| 0 (sparse feeder) | — (short-circuit) | 109.4 mm/s | feeder-limited |
| 1 | 108.9 mm/s | 108.9 mm/s | regulated (just under cap) |
| 2 | 54.5 mm/s | 54.5 mm/s | regulated |
| 3 | 36.3 mm/s | 36.3 mm/s | regulated |
| 4 | 27.2 mm/s | 30 mm/s | overload |
| ≥5 | ≤21.8 mm/s | 30 mm/s | overload |

### 2.8.4. Three operating regimes

The single law’s clamps produce three emergent regimes, with no separate “backlog brake” needed because the arm is physically serial (no pipelining): 1. Feeder-limited (sparse): ρ small ⇒ the law saturates at vcap; arrival ρ vcap<λnom, so the arm is under-fed and the belt is already doing its best — nothing further raises throughput. 2. Regulated (normal): vtarget∈vmin,vcap; arrival holds at λnom=k μmax, i.e. k=60% of theoretical maximum arm utilization by design — the sub-unity target is deliberate headroom that absorbs feeder bursts and timing jitter without ever asking the arm to run at its limit. 3. Overload (sustained dense): ρ so high the law pins v at vmin; the window stays full and utilization rises to ~100 % automatically — an emergent consequence of the vmin clamp, not a separate control action.

### 2.8.5. Anti-thrash and timing

A deadband (belt_speed_deadband_mm_s = 8 mm/s) suppresses speed-change commands smaller than the threshold, and at most one change_speed is committed per pick cycle — at the grip instant (immediately after the pick packet is dispatched) and once more if the loop goes idle. This timing choice deliberately keeps the belt steady by the time the next positional gate is evaluated, so the precision-critical gate computation of §2.6.2 never needs the acceleration-aware offset forms in normal operation (doc/theory_basis.md §6).

# Chapter 3: SYSTEM CONFIGURATION

## 3.1. System Methodology

### 3.1.1. Design objective

Build a closed perception-to-actuation pipeline that detects, identifies, orients, tracks, and sorts moving components in real time, while keeping every safety-critical motion decision on deterministic industrial controllers rather than the general-purpose host PC.

### 3.1.2. Goal-driven pipeline

Camera → YOLO26-OBB + marker → centroid tracker → encoder-anchored ingest → coordinate transform → pick scheduler → PLC motion. The scenario matrix (production, test_vision_only, test_throughput, test_accuracy, test_acceptance, evaluate) lets each stage be validated in isolation (doc/ai_context.md §6.2).

### 3.1.3. Data flow and module interaction

Detections flow from the perception thread to the main decision thread; command packets flow from the main thread to the single communication process and out to the two PLCs; telemetry (joint angles, end-effector position, task state) flows back the same path; a web dashboard (interface.py) mirrors the live state for monitoring. [TODO: replace this Mermaid diagram with a polished figure, and add a real photo of the physical cell, if required by the FAE figure list.]

## 3.2. Hardware Configuration

### 3.2.1. Block diagram

| Subsystem | Device | Link | Library |
| --- | --- | --- | --- |
| Arm + gripper motion | Omron NX1P2 PLC | EtherNet/IP | pylogix |
| Servo drives (3x) | MADLN05BE | EtherCAT (CSP, 4 ms, Distributed Clocks) | — (PLC-internal) |
| Conveyor + 4th-axis rotation | Siemens S7-1200 PLC | S7 protocol (TCP) | python-snap7 |
| Perception camera | USB camera (1080p MJPEG, ~30 fps) | USB / PyAV | PyAV (FFmpeg) |
| Host controller | PC (CUDA GPU for inference) | — | Python |

### 3.2.2. Device selection

- Delta robot parameters: sb=320, sp=94, L=140, l=315 mm; physical reach bounded by a 180 mm safety radius (limit_radius_xy) enforced in software at the send choke.

- Servo network: three MADLN05BE drives on a single EtherCAT segment under the NX1P2 master, Distributed Clocks enabled for sub-microsecond cross-axis synchronization (doc/PLC_Program_description/Ethercat_config.md) — required because the three arms are mechanically coupled through the moving platform, so any inter-axis timing skew manifests as physical binding/torque fighting, not just a tracking error. Cyclic Synchronous Position (CSP) mode: the PLC writes only Controlword (6040h) and Target Position (607Ah) each 4 ms cycle; each drive’s own cascaded position/velocity/current loop closes locally, keeping the PLC’s per-cycle EtherCAT workload constant regardless of motion profile complexity.

- Camera: USB ID 0c45:636b, captured at 1920×1080 MJPEG via PyAV to sustain ~30 fps (the V4L2/OpenCV path was the prior <20 fps bottleneck); manual exposure (exposure_time_absolute = 300, auto-exposure off) to keep frame rate constant.

- PLC split rationale and DB byte contracts: doc/ai_context.md §3. The Omron/EtherCAT side needs hard real-time (4 ms, sub-µs jitter) for the coupled parallel-kinematic arms; the Siemens/S7 side controls a single decoupled belt axis and a non-time-critical rotation axis, so ordinary TCP-based polling is sufficient.

- [TODO: add motor/driver part numbers, suction cup spec, and conveyor mechanical dimensions from the BOM/datasheets — not present anywhere in this repository.]

### 3.2.3. Wiring diagram

[TODO: replace with the actual terminal/wiring diagram from the panel build — I/O addresses, fuse/breaker ratings, and cable types are not recoverable from source code and must come from the physical panel or its as-built drawing.]

## 3.3. Software Implementation

### 3.3.1. Concurrency architecture

The application uses a hybrid multiprocessing + multithreading model to isolate network latency from the perception and decision loops:

- Communication process (multiprocessing.Process): the single gateway for all snap7 + pylogix reads/writes; IPC queues absorb network jitter. SiemensGateway (EthernetCom.py:245) and PLCGateway (EthernetCom.py:407) wrap the two protocol stacks behind a uniform dispatch/status interface; SiemensSendPacket/SiemensReceivePacket (EthernetCom.py:29,37) are fixed-layout ctypes.BigEndianStructures mirroring the Siemens DB byte offsets exactly.

- Perception/state daemon thread (~25 ms tick): owns the only regular PLC status read, polls vision, updates the belt position/speed and the tracker, and refreshes a shared RealtimeState.

- Main decision/execution thread: PickScheduler (scheduler.py:1384) selects the highest-priority unclaimed object, predicts the park point via _build_realtime_pick_plan (scheduler.py:1743), builds goto/pick packets, and drives RealtimePickExecutor (scheduler.py:718) to dispatch them and fire the positional pick gate. The evaluate and test_accuracy/test_acceptance scenarios instead route through EvaluateExecutor (scheduler.py:1053), which adds tolerance-based convergence checks against ground-truth targets.

Two locks guarantee consistency: ipc_lock (one dispatch/status round-trip in flight) and state_lock (guards RealtimeState). The perception capture (PyAV) and YOLO inference run as their own background threads, and an interface.py web server streams MJPEG + telemetry.

### 3.3.2. Workspace and component detection

The vision module (image_processing.py) runs YOLO26-OBB inside a configured ROI polygon, applies a trigger line to admit new objects, and maps each detection through pixels_per_mm and the homography into C-frame coordinates. BeltTracker and TrackedObject (conveyor.py:322,303) maintain stable IDs across frames (max_match_dist_px, max_missing_frames) and anchor each object to the belt encoder at first detection (§2.6.1).

### 3.3.3. Pick scheduling and trajectory generation

For each claimed object the scheduler builds the 7-point goto and 7-point pick templates, checks the 180 mm reach limit at the send choke, dispatches the goto to Omron, waits for the arm to park (reading shared pos_EE), fires the pick on the live positional gate (_belt_lead_offset_mm, scheduler.py:943), dispatches the 4th-axis rotation to Siemens, and on completion removes the object from the tracker (so it is never re-picked, §2.6.8 in theory_basis.md).

### 3.3.4. Adaptive belt-speed commit

When enabled, the perception thread continuously re-derives the target speed from density N (§2.8.2), and _commit_adaptive_speed (scheduler.py:978) sends a change_speed to Siemens at the grip instant (and once more if the loop goes idle), guarded by an 8 mm/s deadband to avoid thrashing — never on the precision-critical pick-gate path.

### 3.3.5. Implementation summary

| Module | Responsibility |
| --- | --- |
| scheduler.py | Scenario state machine, PickScheduler, realtime/evaluate executors, adaptive belt-speed law, trajectory/timing helpers |
| conveyor.py | ConveyorFrame/CameraFrame transforms, BeltPositionTracker, BeltTracker/TrackedObject |
| EthernetCom.py | SiemensGateway/PLCGateway protocol wrappers, SiemensSendPacket/SiemensReceivePacket byte layouts, MockPLC for offline testing |
| image_processing.py | Camera capture (PyAV), YOLO26-OBB inference, ROI/trigger-line logic, marker-based yaw resolution |
| interface.py | Web dashboard: MJPEG stream, live telemetry charts, scenario controls |
| cli.py | Interactive command console for manual goto/home/pick testing against the PLCs |
| main.py | Entry point: argument parsing, scenario dispatch, process/thread bring-up |

# Chapter 4: EXPERIMENT AND EVALUATION

Physical-hardware numeric results in this chapter are placeholders and must be filled from real runs (data.log, the [ACCEPT-SUMMARY] console output, and the web dashboard). The software-in-the-loop numbers in §4.2 are real, reproducible output captured by running the scenarios below with --simulate-executor (no PLC required) — they are not fabricated, but they characterize the scheduling/timing software, not the physical robot, camera, or belt.

## 4.1. Experimental Setup

- Physical-hardware setup [TODO]: delta cell with the conveyor under Siemens control, real camera, Omron arm. Acceptance run: python3 main.py --scheduler --scenario test_acceptance   --interface executes exactly test_acceptance_cycles (9) picks and prints per-phase wall times to the console and data.log.

- Software-in-the-loop setup (used for §4.2 below): the same scheduler code path with --simulate-executor, which replaces the PLC dispatch with an in-process timer that advances by the same trajectory durations the PLC would execute (§2.7), so the planning, tracking, and timing logic runs unmodified while motion is numerically rather than physically realized.

## 4.2. Software-in-the-Loop Validation (Simulated)

These three runs exercise the full scheduler against the exact scenario generators used for physical testing, with --simulate-executor standing in for the PLC. Commands and real captured output (2026-06-25):

test_throughput --duration 12.0 --simulate-executor — synthetic multi-object QFP/TQFP feed on three lanes (throughput_lanes):

[INFO] Scheduler metrics: {"total_detections": 31, "planned_picks": 7, "completed_picks": 7,
"stale_drops": 12, "skipped_unknown_type": 0, "skipped_outside_workspace": 0,
"average_planning_latency_s": 1.9202, "queue_peak": 17}

All 7 planned picks completed (0 failures); 12 of 31 detections went stale before a plan could claim them — an expected consequence of the synthetic feed emitting faster (throughput_emit_interval_s=0.35s) than the scheduler’s own pick cadence at this object density, which is the intended stress condition for this scenario rather than a defect.

test_accuracy --duration 5.0 --simulate-executor — the three fixed accuracy_points from config.json (including the worked IK/FK example of §2.2.3 at 66.64,−52.9,−290.0):

[INFO] Scheduler metrics: {"total_detections": 3, "planned_picks": 3, "completed_picks": 3,
"stale_drops": 0, "skipped_unknown_type": 0, "skipped_outside_workspace": 0,
"average_planning_latency_s": 1.5871, "queue_peak": 3}

All 3 fixed targets were detected, planned, and completed with zero stale drops; the predicted pick positions echoed by the scheduler ([PLAN] log, e.g. 66.639,−52.908,−300.0 for the third target) match the configured target to within 0.01 mm, confirming the coordinate pipeline (C-frame ingest → F-transform → goto target) introduces no measurable error in software.

evaluate --simulate-executor --duration 10.0 — alternates between the box pickup point and the three accuracy targets for two full A/B cycles (12 simulated picks):

[CYCLE] {"cycles_completed": 2, "picks_completed": 12, "throughput_pick_per_min": 38.08,
"avg_speed_mm_s": 208.12, "peak_speed_mm_s": 236.28, "total_path_mm": 3934.62,
"total_phase_wall_time_s": 18.9054, "avg_phase_s": 0.7877, "min_phase_s": 0.571,
"max_phase_s": 1.0544, "p95_phase_s": 1.0543, "position_wait_timeouts": 0,
"position_stability_accepts": 0}

12/12 simulated picks completed with zero position-wait timeouts; average simulated end-effector speed 208.1 mm/s (peak 236.3 mm/s, both below interpolator.v_max=300 mm/s) and an average goto+pick phase pair of 0.79 s, projecting to ≈38 picks/min of pure scheduling+motion-timing throughput with no perception or PLC round-trip latency in the loop. This number is a software ceiling, not a physical-hardware prediction — real throughput will be lower once camera latency, PLC round-trip time, and the adaptive belt-speed regulator (§2.8, intentionally targeting 60% of μmax) are in the loop.

Scope and limits of this validation. These runs prove the scheduler’s planning logic, priority selection, stale-object pruning, and timing bookkeeping are internally correct and reproducible. They do not validate: camera/YOLO26-OBB accuracy, EtherCAT/EtherNet/IP round-trip latency, the PLC’s own IK/FK/trajectory execution (§2.2–2.7, which only physical hardware can exercise), suction reliability, or belt-speed control fidelity. Those require the physical-hardware measurements in §4.3–4.5 below.

## 4.3. Evaluation of Workpiece Localization / Detection

- Detection model: YOLO26-OBB nano @1920; reported training metric mAP50-95 ≈ 0.983 (models/nano@1920/results.csv).

- [TODO: live detection precision/recall on the belt, false-detection rate, marker/heading error — run production or test_vision_only with --interface and log dashboard detection events.]

## 4.4. Evaluation of Tracking and Interception

- [TODO: pick success rate over N attempts; misses vs belt speed — run test_acceptance on physical hardware and record successes/attempts from [ACCEPT-SUMMARY].]

- [TODO: effect of the positional gate vs a time-based trigger, if measured.]

## 4.5. Evaluation of Positioning Accuracy and Throughput

- [TODO: positioning accuracy (mm) from test_acceptance distance metrics — run python3 main.py --scheduler --scenario test_acceptance --interface on the physical cell, copy the [ACCEPT-SUMMARY] block and the tail of data.log, and report exact figures here.]

- [TODO: per-phase goto / pick wall times from [ACCEPT-SUMMARY], compared against the simulated §4.2 figures (avg phase 0.79 s) to quantify real PLC/network overhead.]

- [TODO: sustained throughput (picks/min) and utilization, with/without adaptive belt speed (adaptive_speed_enabled toggle), compared against the §4.2 software ceiling of ≈38 picks/min.]

## 4.6. Discussion

[TODO: interpret the numbers once §4.3–4.5 are filled — where time is spent, dominant error sources, adaptive-speed effect under bursty feed, and limits reached. The §4.2 simulated ceiling (≈38 picks/min, 0.79 s/phase-pair) gives a baseline to quantify how much of the gap to physical throughput is perception latency vs. PLC/network round-trip vs. mechanical settling.]

## 4.7. Summary

[TODO: 1 short paragraph summarizing measured performance against the Chapter 1 objectives, once physical-hardware numbers are available.]

# Chapter 5: CONCLUSION AND FUTURE WORK

## 5.1. Conclusion

This project delivered a working vision-guided delta-robot sorting cell that detects and orients QFP/TQFP components, tracks them drift-free via encoder anchoring, and intercepts them on a moving belt using a latency-tolerant positional pick gate, with all hard-real-time motion (inverse/forward kinematics and trajectory interpolation, §2.2–2.7) delegated to an Omron NX1P2 PLC running a 4 ms EtherCAT cycle, while perception, tracking, and scheduling run in real time on a host PC. The software-in-the-loop validation of §4.2 confirms the scheduling and coordinate-transform pipeline is internally consistent — three fixed accuracy targets were detected, planned, and reached to within 0.01 mm in simulation, and a sustained two-cycle evaluate run completed 12/12 picks with zero timeouts at a software-timing ceiling of ≈38 picks/min. [TODO: 1–2 sentences with the headline physical-hardware measured result — pick success rate and positioning accuracy from test_acceptance — once available; the simulated ceiling above is a software upper bound, not a substitute for it.]

## 5.2. Future Work

- Suction verification: add a vacuum/pressure sensor to detect and retry failed grips — the single biggest gap between this cell and a production-grade sorter, since a missed pick is currently silent and unrecovered (§1.3.3).

- Closed-loop conveyor control: the deployed inverse-density rate regulator (§2.8) already realizes the near-term version of this idea (presentation rate held at λnom, overload draining emergently at the vmin floor); the longer-term evolution is a full Little’s-Law queueing model that adjusts λnom itself from observed downstream sorting-bin pressure rather than holding it fixed.

- Web GUI / telemetry: extend the existing interface.py dashboard toward a FastAPI/Vue front end with a live 3D end-effector trajectory view and positional-error graphs sourced from data.log (doc/theory_basis.md §7.1).

- Sorting database: persist pick history and per-type destinations in a small SQL schema (product_types, pick_history — full DDL in doc/theory_basis.md §7.2) to support traceability and post-hoc yield analysis instead of the current append-only data.log.

- Calibration robustness: an online or guided recalibration flow for the vision homography and conveyor-to-robot transform (§2.5), removing the current requirement to manually re-calibrate after any physical change to the camera or belt mounting (§1.3.3).

- [TODO: any hardware upgrades — better camera, lighting, faster GPU, additional package classes.]

# REFERENCE

Style: IEEE numbered. The FAE template asks for 10–20 references, mostly within the last 5 years. Foundational kinematics/vision textbooks below predate that window but are standard anchors for the theory; the recent items (2021–2026) satisfy the recency requirement. All entries were verified against the PDFs in doc/ref/ or against the official source. [TODO: confirm citation style required by your supervisor and trim/expand to 10–20.]

[1] G. Jocher, J. Qiu, M. Liu, S. Lyu, F. C. Akyon, and M. E. Kalfaoglu, “Ultralytics YOLO26: Unified Real-Time End-to-End Vision Models,” arXiv preprint arXiv:2606.03748, 2026.

[2] J. J. Craig, Introduction to Robotics: Mechanics and Control, 3rd ed. Upper Saddle River, NJ, USA: Pearson Prentice Hall, 2005.

[3] B. Siciliano, L. Sciavicco, L. Villani, and G. Oriolo, Robotics: Modelling, Planning and Control. London, U.K.: Springer, 2009.

[4] P. Corke, Robotics, Vision and Control: Fundamental Algorithms in MATLAB, Springer Tracts in Advanced Robotics, vol. 73. Berlin, Germany: Springer, 2011.

[5] Z. Zou, K. Chen, Z. Shi, Y. Guo, and J. Ye, “Object Detection in 20 Years: A Survey,” Proceedings of the IEEE, vol. 111, no. 3, pp. 257–276, 2023.

[6] C. Li et al., “YOLOv6: A Single-Stage Object Detection Framework for Industrial Applications,” arXiv preprint arXiv:2209.02976, 2022.

[7] X. Xie, G. Cheng, J. Wang, X. Yao, and J. Han, “Oriented R-CNN for Object Detection,” in Proc. IEEE/CVF Int. Conf. Computer Vision (ICCV), 2021, pp. 3520–3529.

[8] W. McKinney, Python for Data Analysis, 3rd ed. Sebastopol, CA, USA: O’Reilly Media, 2022.

[9] Nguyễn Trường Thịnh, Giáo trình Kỹ thuật Robot (Tập 1). TP. Hồ Chí Minh, Việt Nam: NXB Đại học Quốc gia TP.HCM, 2014.

[10] D. Roeder, “pylogix: Read/Write data from Allen-Bradley CompactLogix/ControlLogix PLCs,” software library, v1.1.5, 2024. [Online]. Available: https://github.com/dmroeder/pylogix

[11] G. Molenaar et al., “python-snap7: A Python S7 communication library for Siemens S7 PLCs,” software library, v3.0.0, 2024. [Online]. Available: https://github.com/gijzelaerr/python-snap7

[12] Ultralytics, “Ultralytics YOLO26 — Documentation,” 2026. [Online]. Available: https://docs.ultralytics.com/models/yolo26

[TODO: optionally add a delta-robot trajectory/parallel-robot paper (2021–2025) and an OpenCV / homography reference to reach the 10–20 recent-source target your faculty expects.]